Thermal & Optical Bi-spectrum Network Bullet Camera

SS-222 (9266)

High sensitivity thermal module with 256 x 192 resolution NETD is less than 60 mk (@25° C, F#=1.0)

Supports contrast adjustment

Leading thermal image processing technology: Adaptive AGC, DDE, 3D DNR Up to 15 palettes of adjustable color

Reliable temperature-anomaly alarm Temperature Range From -15°C to +150°C High quality optical module with 2 MP resolution

Bi-spectrum image fusion, picture-in-picture preview

Specification

| Thermal |  |
| --- | --- |
| Image Sensor | VOx Uncooled Focal Plane Arrays |
| Resolution | 256×192 |
| Pixel Interval | 12μm |
| NETD | Less than 60 mK (@25°C,F#=1.1) |
| Aperture | F1.0 |
| Field of View | 35° × 27° (H × V) |
| Optical |  |
| Image Sensor | 1/2.8" 2.0M Pixel CMOS |
| Resolution | 1920×1080P |

| Min. Illumination | Color: 0.005Lux @ (F1.2, AGC ON), B/W: 0.001 Lux @ (F1.2, AGC ON) |
| --- | --- |
| Field of View | 84° × 45° (H × V) |
| Focal Length | 4mm |
| Shutter Speed | 1s to 1/100,000s |
| White Balance | Auto/Manual/ATW (Auto-tracking White Balance)/Indoor/Outdoor/Daylight Lamp/Sodium Lamp |
| Day & Night | ModeIR cut filter with auto switch |
| WDR | 80 dB |
| Feature |  |
| Bi-spectrum Image Fusion | Fusion view of thermal view and overlaid details of the optical channel |
| Picture in Picture | Combines details of thermal and optical image PIP, overlay thermal image on optical image |
| Smart Function |  |
| Face snapping | Built-in deep learning AI algorithm, Supports simultaneous detection of 20-30 faces |
| Temperature Measurement | Support global and local temperature |
| Temperature Range | From -15°C to +150°C |
| Temperature Accuracy | Target temperature 35°C ^ 38°C ±0.3 °C Target temperature 20°C ^ 33°C ±0.6 °C Target temperature 38°C ^ 50°C ±0.6 °C |
| Network |  |
| Main Stream | Thermal: 25fps(1920 × 1080, 1280 × 720) |
| Sub Stream | Thermal: 25fps(704 × 576, 352 × 288) |
| Video Compression | H.264 (Baseline/Main/High Profile) /MJPEG/H.265 |
| Audio Compression | G .711u/G.711a/G.722.1/MP2L2/G.726/PCM |
| Protocols | TCP/IP, ONVIF, GB/T 28181, DHCP, RTP, RTSP, PPPoE, UPnP, UDP |
| API | ONVIF (Profile S, Profile G, Profile T), SDK |
| General |  |
| Web Client Language | languages English, Chinese |
| Power | DC 12V, 0.65A |
| Work Temperature/Humidity | From -20°C to 55°C; Humidity: 95% or Less |
| Protection Level | IP67 |
| Dimension | 246 mm × 101 mm × 81 mm (with bracket) |
| Weight | Approx. 1.0 kg |