- Enter TCP/IPv4 setting, change PC IP to 192.168.1.230 / 192.168.20.230 (both domain has to be added)

- NVR-PC connected directly

- Start tftp32

Go to “Setting” and make sure the directory is set to the path where your files are located

- Press the button on the main board and do not let go, power on the NVR

- After seeing data progress bar on tftp32 then can release the button. This process takes about 5mins and NVR will reboot which means the whole progress is done.